Compile, upload and run the code one time with the “Pre.ino” one .

This will result in the nano writing 16 patches to its EEPROM.

One patch will contain a pattern, the others are empty patches.

Next use the “After.ino” and compile, upload and run the code. This will program the Euclidean sequencer code into the nano which will be run each time you start up the nano.

The nano will read the patches from memory each time it starts and chooses the patch you saved last.

Upon startup the sequencer is in 'Pattern Length / Tempo Mode'.

When pressing the rotary encoder you can switch to the second mode, 'Num Pulse Mode'.

Press it again and the mode is 'Rotate Mode'.

Press again and the mode returns to 'Pattern Length / Tempo Mode'.

In 'Pattern Mode' rotating the encoder will increase or decrease the pattern length of the chosen trigger channel. Each time you change the length of the pattern a new fitting Euclidean pattern is generated depending on the number of pulses chosen (see below) for this pattern. Rotating the dial with the tap tempo / shift button pressed at the same time, will allow for changing tempo of the sequencer's internal clock.

In 'Num Pulse Mode' the encoder can be used to set the number of trigger pulses for the chosen trigger channel. The pulses will follow a Euclidean pattern.

In 'Rotate Mode' it is possible to shift the trigger pulse pattern accross the total length of the pattern.

Pressing the shift button and the rotary switch at the same time (…fill mode) will select the 'Program Mode'.

Using the rotary encoder's knob 1 of 16 memory cells can be selected.

- Pressing the trigger channel D button will store the current pattern of the selected trigger channel to the corresponding memory cell.

- Pressing the trigger channel A button will recall the patch pattern stored in the EEPROM memory cell and store it in the sequencer's active memory at the patch number selected with the rotary dial.

- Pressing the trigger channel B button will clear the patch in the eeprom the rotary encoder is set to. The pattern already in the working memory however will not be changed. Don't forget to save it if you like it. The LEDs will light up red for the locations containing a patch and green for the locations that are cleared. When in 'Program Mode' pressing the rotary switch will switch to 'Pattern Length / Tempo Mode'

To exit Programm mode press the main encoder.

Changed the slowest loop time to 2 sec per step for more slower beats possible.